# TAS58xx EQ Control Proposal

Simpler EQ control for the tas58xx component. Proposal by @mrtoy-me for review and comment.

# Summary

eq_mode becomes a switch (On / Off). On at first boot, then the last setting is restored.

Three EQ band lists: stereo_bands (one slider drives both channels), left_bands, right_bands.

Bands, EQ presets and fixed filters can all be combined on a channel, up to 15 biquads per channel.

EQ Off: bands and presets are flat. Fixed filters are flat too, unless fixed_filters_always_on: true.

Removed: the Right=Left, Left On, Right On and BiAmp modes, and the rule against bands and a preset on the same channel.

# YAML

audio_dac:

- platform: tas58xx

id: dac1

left_fixed_filters:                 # optional, no slider

- eq_filter: notch

frequency: 50Hz

q_factor: 5

right_fixed_filters:

- eq_filter: band_pass

frequency: 1kHz

q_factor: 2

fixed_filters_always_on: false      # optional, default false

switch:

- platform: tas58xx

eq_mode:

name: EQ

restore_mode: RESTORE_DEFAULT_ON  # default

select:

- platform: tas58xx

eq_preset_left_channel:             # optional, biquads 1-3

name: Left Preset

eq_preset_right_channel:

name: Right Preset

number:

- platform: tas58xx

stereo_bands:                       # one slider, both channels

- name: Bass

eq_filter: low_shelf

frequency: 100Hz

- name: 1kHz

frequency: 1kHz

left_bands:                         # left only

- name: Left 120Hz

frequency: 120Hz

right_bands:                        # right only

- name: Right 3kHz

eq_filter: peaking

frequency: 3kHz

q_factor: 4

Band options (eq_filter, frequency, q_factor, gain limits) and fixed filter options are as they are now.

# EQ Switch

|  | EQ On | EQ Off, always_on false (default) | EQ Off, always_on true |
| --- | --- | --- | --- |
| Bands (stereo, left, right) | active | flat | flat |
| EQ presets | active | flat | flat |
| Fixed filters | active | flat | active |
| How Off is done | - | DSP bypass flag: one write, no click | band and preset biquads written as all pass |

A gain or preset changed while EQ is Off is kept and takes effect when EQ is turned On.

The switch is required when bands or presets are defined, or fixed filters with fixed_filters_always_on: false. Otherwise a warning says it is not used.

# Biquads on Each Channel

An EQ preset, if there is one, uses biquads 1-3.

Then stereo bands, the channel's own bands and its fixed filters, cuts first: high pass, then low pass / notch / band pass, then shelves / equalizer / peaking. YAML order is kept within a group, stereo bands first.

Preset + stereo bands + channel bands + fixed filters must be 15 or fewer per channel, otherwise a config error.

A stereo band can sit at a different biquad on each channel; its slider writes both.

# Examples

## Stereo

Both speakers get the same EQ from one set of sliders (replaces Right=Left).

number:

- platform: tas58xx

stereo_bands:

- name: 25Hz

frequency: 25Hz

# ... ISO 2/3-octave centres ...

- name: 16kHz

frequency: 16kHz

## Bi-amp (woofer left, tweeter right)

select:

- platform: tas58xx

eq_preset_left_channel:

name: Woofer crossover            # LF preset

eq_preset_right_channel:

name: Tweeter crossover           # HF preset

number:

- platform: tas58xx

stereo_bands:                       # tone for the whole speaker

- name: Bass

eq_filter: low_shelf

frequency: 100Hz

- name: Treble

eq_filter: high_shelf

frequency: 8kHz

left_bands:

- name: Woofer 300Hz

frequency: 300Hz

right_bands:

- name: Tweeter 5kHz

frequency: 5kHz

| Biquad | Left (woofer) | Right (tweeter) |
| --- | --- | --- |
| 1-3 | LF crossover preset | HF crossover preset |
| 4 | Bass | Bass |
| 5 | Treble | Treble |
| 6 | Woofer 300Hz | Tweeter 5kHz |

Warning: EQ Off flattens the crossover presets, so the tweeter gets full range. This will be in the README.

# PBTL

Only the left channel is output, so stereo_bands apply to the left. Right bands, the right preset and right fixed filters give a "not used" warning.

# Breaking Changes

eq_mode moves from select: to switch: and only has On and Off.

YAML that relied on Right=Left changes left_bands to stereo_bands.

The saved EQ mode is reset once after updating.

# YAML Format Notes

Band and fixed filter lists accept either YAML form; both are valid and can be mixed:

left_bands:

- name: Left 100Hz

frequency: 100Hz

- {name: Left Bass, eq_filter: low_shelf, frequency: 80Hz}

Each band is a full ESPHome number, so standard options such as id, icon, mode and entity_category can be added. Commas without { } are invalid.