# TAS58xx EQ Control Proposal

A flexible, powerful and simpler EQ control for the tas58xx component. Proposal by @mrtoy-me for review and comment.

# Summary

- eq_mode becomes a switch (On / Off). On at first boot, then the last setting is restored.

- Three EQ band lists: stereo_bands (one slider drives both channels), left_bands, right_bands.

- Bands, EQ presets and fixed filters can all be combined on a channel, up to 15 biquads per channel.

- EQ Off: bands and presets are flat. Fixed filters are flat too, unless fixed_filters_always_on: true.

# YAML

audio_dac:

- platform: tas58xx

id: dac1

left_fixed_filters:                 # optional, no slider

- eq_filter: notch

frequency: 50Hz

q_factor: 5

right_fixed_filters:		     # optional, no slider

- eq_filter: band_pass

frequency: 1kHz

q_factor: 2

fixed_filters_always_on: false      # optional, default false

switch:

- platform: tas58xx

eq_mode:

name: EQ

restore_mode: RESTORE_DEFAULT_ON  # default

select:

- platform: tas58xx

eq_preset_left_channel:             # optional, consumes left biquads 1-3

name: Left Preset

eq_preset_right_channel:	     # optional, consumes right biquads 1-3

name: Right Preset

number:

- platform: tas58xx

stereo_bands:                       # one slider, both channels

- name: Bass

eq_filter: low_shelf

frequency: 100Hz

- name: 1kHz

frequency: 1kHz

left_bands:                         # left only

- name: Left 120Hz

frequency: 120Hz

right_bands:                        # right only

- name: Right 3kHz

eq_filter: peaking

frequency: 3kHz

q_factor: 4

# EQ Switch

|  | EQ On | EQ Off, always_on false (default) | EQ Off, always_on true |
| --- | --- | --- | --- |
| Bands (stereo, left, right) | active | flat | flat |
| EQ presets | active | flat | flat |
| Fixed filters | active | flat | active |
| How Off is done | - | DSP bypass flag: one write, no click | band and preset biquads written as all pass |

- A gain or preset changed while EQ is Off is kept and takes effect when EQ is turned On.

- The switch is required when bands or presets are defined, or fixed filters with fixed_filters_always_on: false. Otherwise a warning says it is not used.

# Biquads on Each Channel

- An EQ preset, if there is one, uses biquads 1-3.

- Then stereo bands, the channel's own bands and its fixed filters, cuts first: high pass, then low pass / notch / band pass, then shelves / equalizer / peaking. YAML order is kept within a group, stereo bands first.

- Preset + stereo bands + channel bands + fixed filters must be 15 or fewer per channel, otherwise a config error.

- A stereo band can sit at a different biquad on each channel; its slider writes both.

# Examples

## Stereo

Both speakers get the same EQ from one set of sliders (replaces Right=Left).

number:

- platform: tas58xx

stereo_bands:

- name: 25Hz

frequency: 25Hz

# ... ISO 2/3-octave centres ...

- name: 16kHz

frequency: 16kHz

## Bi-amp (woofer left, tweeter right)

select:

- platform: tas58xx

eq_preset_left_channel:

name: Woofer crossover            # LF preset

eq_preset_right_channel:

name: Tweeter crossover           # HF preset

number:

- platform: tas58xx

stereo_bands:                       # tone for the whole speaker

- name: Bass

eq_filter: low_shelf

frequency: 100Hz

- name: Treble

eq_filter: high_shelf

frequency: 8kHz

left_bands:

- name: Woofer 300Hz

frequency: 300Hz

right_bands:

- name: Tweeter 5kHz

frequency: 5kHz

| Biquad | Left (woofer) | Right (tweeter) |
| --- | --- | --- |
| 1-3 | LF crossover preset | HF crossover preset |
| 4 | Bass | Bass |
| 5 | Treble | Treble |
| 6 | Woofer 300Hz | Tweeter 5kHz |

Warning: EQ Off flattens the crossover presets, so the tweeter gets full range. This will be in the README.

# PBTL

Only the left channel is output, so stereo_bands apply to the left. Right bands, the right preset and right fixed filters give a "not used" warning.

# Breaking Changes

- eq_mode moves from select: to switch: and only has On and Off.

- Number EQ bands are defined as stereo_bands, left_bands and right_bands lists.

# YAML Format Notes

Band and fixed filter lists accept either YAML form; both are valid and can be mixed:

left_bands:

- name: Left 100Hz

frequency: 100Hz

- {name: Left Bass, eq_filter: low_shelf, frequency: 80Hz}

Each band is a full ESPHome number, so standard options such as id, icon, mode and entity_category can be added. Commas without { } are invalid.