# TAS58xx PPC 2.0 Mode Review: Register 0x46 and Biquad Verification

Oct 16, 2026 · @mrtoy-me

## Summary

The esphome-tas58xx startup sequences and biquad helper match TI PurePath Console (PPC) output for both chips; keep 96 kHz as the biquad sample rate.

- Scope: TAS5805M and TAS5825M, 2.0 (stereo) process flows only, at 96 kHz and 48 kHz.

- Startup: both drivers reproduce PPC's ROM-flow startup register for register, apart from deliberate additions (ANA_CTRL, FAULTZ pin, CBC).

- Register 0x46: on TAS5825M it is the documented DSP_CTRL register and 0x11 selects 96 kHz processing. On TAS5805M it is undocumented; PPC writes 0x01 for the 96 kHz flow.

- Biquads: the proposed make_biquad() helper matches PPC to the bit for Equalizer, Low Shelf, Butterworth high and low pass at both rates, with one 1 LSB PPC rounding difference. PPC's Bass Shelf is a separate filter the helper does not implement.

## Method

Six PPC register dumps were exported with the same test filters and compared against both drivers and TI's documentation.

| Chip | PPC process flow | Rate | Dump file |
| --- | --- | --- | --- |
| TAS5805M | 3-Band DRC 2.0 | 96 kHz | hz50_tas5805_dump.cfg |
| TAS5805M | 3-Band DRC & FIR 2.0 (FIR bypassed) | 48 kHz | hz50_tas5805_dump_48k.cfg |
| TAS5825M | 2-Band DRC & AGL 2.0 | 96 kHz | HZ50-tas5825_dump.cfg |
| TAS5825M | Base/Pro 2.0 | 48 kHz | hz50_tas5825_dump_48k.cfg |
| TAS5825M | Base-FIR 2.0 (FIR bypassed) | 48 kHz | hz50_tas5825_dump_48k base-fir.cfg |
| TAS5825M | Base/Pro Hybrid 2.0 | 48 kHz | hz50_tas5825_dump_48k base-pro-hybrid.cfg |

Test filters (left channel): BQ1 Equalizer (Q factor), then High Pass and Low Pass Butterworth 2, Low Shelf (TAS5805M) or Bass Shelf (TAS5825M). All at 50 Hz, -10 dB, Q 0.71; the remaining biquads were left as all pass.

Compared against:

- ESPHome 2026.10.0b4: tas58xx.cpp init_() and model_tas5805m.cpp / model_tas5825m.cpp.

- esphome-tas58xx: startup tables, DSP address constants and the proposed make_biquad() in tas58xx_helpers.cpp.

Tooling:

make_biquad() is the proposed C++ function, in tas58xx_helpers.cpp, that calculates biquad coefficients on the device. It is planned for a further PR to the ESPHome tas58xx component and is not part of that component yet.

- A script tracked book and page selects through each dump and listed every book 0 control-register write in order.

- make_biquad() was ported line for line to Python (double precision, std::round) and its 5.27 output compared to PPC's bytes.

- TI's biquad.model.js from PPC was read to identify filter formulas, including Bass Shelf.

The C++ itself was not compiled or run for this review; the Python port mirrors it.

References: TAS5805M datasheet (SLASEH5D), TAS5825M datasheet (Rev. H), TAS5805M, TAS5806M and TAS5806MD Process Flows (SLOA263A).

## Startup sequence comparison

For the two 96 kHz ROM flows, ESPHome 2026.10.0b4 and esphome-tas58xx write the same startup registers as PPC; every difference is deliberate.

Shared reset header (all PPC exports and both drivers): page 0, book 0, 03=02 Hi-Z, 01=11 reset, 03=02 Hi-Z, 5 ms delay.

| Chip and flow | PPC writes | Drivers write | Effect |
| --- | --- | --- | --- |
| TAS5805M 96k | 03=00, 46=01, 03=02, 61=0B, 60=01, 7D=11, 7E=FF, p1 51=05 | identical | match |
| TAS5805M 96k | register tuning 53=00 | 53=60 | deliberate: 175 kHz bandwidth with Fsw 768 kHz |
| TAS5825M 96k | reset straight into 7D, no second Hi-Z or delay | adds 03=02, 5 ms delay, 03=00 | ends in deep sleep, the post-reset state PPC starts from; safer |
| TAS5825M 96k | 7D=11, 7E=FF, p1 51=05, p2 1D=00 19=80, 46=11, 53=01, 03=02 | identical | match |
| TAS5825M 96k | 5 ms delay before DSP coefficients | no delay | fine: DSP writes happen later, once I2S runs |
| TAS5825M 96k | tuning 60=02, 62=09 | 60=02, 62=0B, 77=07 | GPIO1 output as FAULTZ, plus cycle-by-cycle current limit |
| Both | 02 DEVICE_CTRL_1, 54 AGAIN, 4C DIG_VOL, 03=03, 78=80 | written from YAML by the driver | intended |

48 kHz TAS5825M flows are not usable with the minimal startup. All three reset to 03=12 (deep sleep, DSP off), write 48=0C, select RAM mode (40=00) and download a DSP program into books 0x64 and 0x78. The Hybrid flow also writes 02=02 and 53=00.

DSP addresses agree with the drivers in every flow except TAS5825M Base-FIR:

- Input mixer: TAS5805M page 0x29 reg 0x18; TAS5825M page 0x0B reg 0x14.

- Channel volume: TAS5805M page 0x2A regs 0x24/0x28; TAS5825M page 0x0B regs 0x0C/0x10. Base-FIR moves it to page 0x13 regs 0x20/0x24.

- First EQ biquad (book 0xAA): TAS5805M page 0x24 reg 0x18; TAS5825M page 0x01 reg 0x30.

- TAS5825M Gang EQ / Bypass EQ: page 0x0B regs 0x28/0x2C.

## Register 0x46

On TAS5825M, 0x46 is DSP_CTRL and sets the DSP processing rate; the driver's 0x11 selects 96 kHz. On TAS5805M it is undocumented, so only PPC's values can be quoted.

| Chip | PPC flow | Rate | 0x46 | Program |
| --- | --- | --- | --- | --- |
| TAS5805M | 3-Band DRC 2.0 | 96 kHz | 0x01 | ROM |
| TAS5805M | 3-Band DRC & FIR 2.0 | 48 kHz | 0x11 | ROM |
| TAS5825M | 2-Band DRC & AGL 2.0 | 96 kHz | 0x11 | ROM (0x40 not written, reset = ROM mode 1) |
| TAS5825M | Base/Pro 2.0 | 48 kHz | 0x09 | RAM download (40=00) |
| TAS5825M | Base-FIR 2.0 | 48 kHz | 0x09 | RAM download (40=00) |
| TAS5825M | Base/Pro Hybrid 2.0 | 48 kHz | 0x09 | RAM download (40=00) |

TAS5825M DSP_CTRL (datasheet section 9.6.1.16, reset 0x01):

| Bits | Field | Values |
| --- | --- | --- |
| 7-5 | reserved |  |
| 4-3 | USER_DEFINED_PROCESSING_RATE | 00 input, 01 48k, 10 96k, 11 192k |
| 2 | reserved |  |
| 1 | BOOT_FROM_IRAM |  |
| 0 | USE_DEFAULT_COEFFS |  |

- 0x11 = 1 0001: 96 kHz processing, default coefficients.

- 0x09 = 0 1001: 48 kHz processing, default coefficients.

- DSP_PGM_MODE (0x40) MODE_SEL: 00000 RAM, 00001 ROM mode 1 (reset), 00010 ROM 2, 00011 ROM 3. The datasheet defines a Hybrid Flow as RAM and ROM components combined.

TAS5805M: 0x46 is absent from the register map, and unlisted addresses are reserved. The TAS5825M layout does not carry over: read that way, 0x01 would be "input rate" and 0x11 "96k", the reverse of the flows. Empirically, 0x01 goes with the 96 kHz flow and 0x11 with the 48 kHz flows; every other control write is identical between them.

## Biquad coefficient verification

make_biquad() reproduces PPC's coefficients exactly for every filter type it implements, at both 96 kHz and 48 kHz. Conditions: 50 Hz, -10 dB, Q 0.71, 5.27 fixed point.

| Filter | Helper function | 96 kHz | 48 kHz |
| --- | --- | --- | --- |
| Equalizer (Q factor) | equalizer_filter | exact (TAS5805M, TAS5825M) | exact (TAS5805M, TAS5825M) |
| High Pass Butterworth 2 | pass_filter(high) | exact (TAS5805M, TAS5825M) | exact (TAS5805M, TAS5825M) |
| Low Pass Butterworth 2 | pass_filter(low) | exact (TAS5825M) | exact (TAS5805M, TAS5825M) |
| Low Shelf | shelf_filter(low) | a1 +1 LSB (TAS5805M) | exact (TAS5805M) |
| Bass Shelf | not implemented | differs (TAS5825M) | not in dumps |

Example, Equalizer at 96 kHz, both chips and helper: 07f5de4f f01da7fe 07ec7f46 0fe25802 f81da26b (b0 b1 b2 a1 a2).

- PPC applies the gain to pass filters. The Butterworth coefficients carry the -10 dB, and pass_filter() does the same, which is why they match.

- Low Shelf 1 LSB is PPC's rounding. TI's own lowShelfCalc gives a1 x 2^27 = 267610483.616, which rounds to 0ff36974 as the helper writes; PPC wrote 0ff36973. One LSB is about 7.5e-9 and inaudible.

- Bass Shelf is a different filter, not another name for Low Shelf. biquad.model.js implements it as bassShelfCalc(), a gainF/gainD/gainN design with a = tan(pi(f/Fs - 0.25)) and no Q. A Python port matched the dump exactly: 07fc83c9 f00feb47 07f395f1 0ff00f50 f80fe0de.

- Not compared: the DRC crossover filters (TAS5805M Low/Mid/High F use a different scaling; TAS5825M DRC High BQ1 is Linkwitz-Riley 2, not in the helper).

## DSP processing rate for biquad calculation

Use 96 kHz for biquad coefficients on both chips, as EQ_SAMPLE_RATE = 96000 already does: coefficients must match the DSP's processing rate, not the I2S input rate.

| Chip | Flow in use | How the DSP rate is set | DSP rate with 48 kHz I2S |
| --- | --- | --- | --- |
| TAS5805M | 3-Band DRC 2.0 96k (ROM) | built-in SRC converts 32-96 kHz input to 88.2 or 96 kHz (SLOA263A) | 96 kHz |
| TAS5825M | 2-Band DRC & AGL 2.0 96k (ROM) | DSP_CTRL 0x11 = 96 kHz; integrated SRC converts input to that target | 96 kHz |

- All example YAMLs use sample_rate: 48000. Neither startup writes SIG_CH_CTRL (0x28), so FS_MODE stays at auto detection.

- 48 kHz processing is not an option here. On TAS5805M it means the FIR flows (no SRC, no 88.2/96 kHz input). On TAS5825M every 48 kHz flow needs a RAM program download, and Base-FIR moves channel volume.

- 44.1 kHz caveat, TAS5805M: the SRC targets "88.2 or 96 kHz", so 44.1 kHz-family input is likely processed at 88.2 kHz. Biquads computed for 96 kHz would then sit about 8% low (1000 Hz lands near 919 Hz).

- 44.1 kHz caveat, TAS5825M: the FSMODE description says the mode is set manually if the input is 44.1/88.2/176.4 kHz. Auto detection may not be enough for that family.

- No register reads back the DSP rate. FS_MON (0x37) reports the detected input rate, but its codes cover 48k and 96k with 44.1k unlisted, so it cannot tell 44.1k from 48k. The only direct check is acoustic: a narrow +12 dB EQ peak at 1 kHz, measured with tones or a sweep.

## Recommendations

Keep the current startup values and the 96 kHz biquad rate; the changes below are comments and documentation, plus two optional safeguards.

- Keep 0x46 as is: TAS5805M 0x01, TAS5825M 0x11. Both match PPC's 96 kHz ROM flows.

- Update the 0x46 comments in the startup tables (tas58xx_minimal.h / tas58xx_startup.h), replacing the earlier "for 48KHz use 0x11" note:

  - TAS5825M: {0x46, 0x11},  // DSP_CTRL: USER_DEFINED_PROCESSING_RATE = 96k (bits 4-3 = 10), USE_DEFAULT_COEFFS

  - TAS5805M: {0x46, 0x01},  // undocumented; PPC 96kHz process flow value (48kHz FIR flows write 0x11)

- Keep EQ_SAMPLE_RATE = 96000. It matches the DSP rate of both ROM flows with 48 kHz I2S.

- Keep make_biquad() unchanged. It is bit-exact against PPC; the Low Shelf 1 LSB is PPC's rounding, not the helper's.

- Document 48 kHz I2S as the supported input in the README and example YAMLs, with the 44.1 kHz caveats above.

Optional safeguards

- ☐ Log a warning when FS_MON (0x37) reports neither 48k nor 96k once audio starts. Catches an unsupported sample_rate, not 44.1 kHz.

- ☐ Acoustic check of the DSP rate: +12 dB narrow peak at 1 kHz, measured at 48 kHz and 44.1 kHz input on TAS5805M.